# T0.x — Tool Availability Inventory

## Assumption

The P0 environment requires multiple FPGA, numerical, simulation, embedded-Linux, and supporting development tools. The purpose of this check was to establish which required components are already available on the development machine before any installation or remediation is attempted.

Availability was assessed from the existing installation locations and command-line checks already performed. Functional verification of the tools is recorded separately where required.

## Method

The existing Xilinx installation tree was inspected from Command Prompt using:

dir "D:\xilinx" /ad

The Vivado installation directory was then inspected using:

dir "D:\xilinx\Vivado" /ad

The existing Vivado 2019.1 installation was checked for simulator components using:

dir "D:\xilinx\Vivado\2019.1\bin" | findstr /i "sim"

The presence of the current unified AMD/Xilinx software platform was checked using:

where /r D:\ xilinx\vitis.exe

The embedded Linux build-system command was checked using:

where petalinux-config

Previously recorded MATLAB and Vivado checks were retained as part of the same environment inventory and were not repeated.

## Result

The following directories were present under D:\xilinx:

.xinstall

DocNav

SDK

Vivado

xic

The D:\xilinx\Vivado directory contained only:

2019.1

The Vivado 2019.1 bin directory contained the simulator executables:

xsim

xsim.bat

The recursive search for Vitis returned:

INFO: Could not find files for the given pattern(s).

The check for the PetaLinux configuration command returned:

INFO: Could not find files for the given pattern(s).

The previously established numerical environment consists of MATLAB R2026a Update 3 with the relevant HDL products installed. The MATLAB hardware-support-package query reported:

No support packages installed.

Vivado 2019.1 and the Classic SDK are already documented separately in the environment record.

## Conclusion

The availability check establishes the following initial state:

- Legacy Vivado 2019.1: Present

- Classic SDK: Present

- XSim simulator: Present

- MATLAB R2026a and HDL products: Present

- MATLAB hardware support packages: Not installed

- Current Vivado/Vitis unified platform: Not found

- vitis.exe: Not found on the verified D: installation tree

- PetaLinux command: Not found

- Vivado HLS: Present as a separate tool; not counted as the current unified platform

- Second-vendor FPGA suite: Not yet established

- Component framework: Not yet established

- Programming cable/driver: Not yet established

No software was installed or modified during this availability check.

## Evidence

The following evidence is to be retained:

- t0_1_vivado_2019_1_installation.png — legacy Vivado installation/version evidence

- t0_1_xilinx_directory.png — available Xilinx installation directories

- t0_1_current_vivado_vitis_not_found.png — current Vivado/Vitis search result

- t0_4_petalinux_command_not_found.png — PetaLinux command search result

- Existing MATLAB evidence recorded under T0.3

- XSim presence output showing xsim and xsim.bat

## Status

Tool availability/inventory check: COMPLETED

Functional verification of the available tools: PENDING

Installation/remediation of missing components: NOT STARTED

# P0 Environment Record

## T0.1 — Legacy FPGA Design Suite

### Assumption

The legacy FPGA design suite required for the assignment is installed on the development machine and can be used for the P0 baseline work.

### Method

The installed Xilinx/Vivado directories were inspected to locate the Vivado installation. The executable was found under:

D:\xilinx\Vivado\2019.1\bin\vivado.bat

Vivado was launched from the command prompt using the installation path rather than relying on the system PATH.

The command used was:

cmd.exe /c "D:\xilinx\Vivado\2019.1\bin\vivado.bat"

The startup output was then inspected for the installed version and build information.

### Result

Vivado launched successfully and reported:

Vivado v2019.1 (64-bit)

The startup output also reported the software and IP build information.

### Conclusion

The legacy Vivado installation is present and executable from the recorded installation path.

The installed version is confirmed as Vivado 2019.1.

The remaining T0.1 information — including the required edition, device support, update level, and the other FPGA design-suite version — has not yet been established and will be recorded separately.

## T0.2 — Licensing Mechanism and Vivado Tier

### Assumption

The installed Vivado environment uses a specific Vivado edition/tier that determines the available device and software-feature coverage.

### Method

The installed Vivado environment was investigated using the Xilinx Add Design Tools or Devices installation utility.

The utility was opened after Vivado 2019.1 had already been installed. The Select Edition screen was inspected without changing the installation.

### Result

The installation utility explicitly identifies the Currently Installed Edition as:

Vivado HL WebPACK

The same screen describes Vivado HL WebPACK as the no-cost, device-limited version of Vivado HL Design Edition.

The screen also shows Vivado HL Design Edition and Vivado HL System Edition as upgrade options.

This provides direct evidence of the installed Vivado edition.

### Conclusion

The installed Vivado 2019.1 environment is confirmed to be:

Vivado HL WebPACK

Therefore, the earlier observation of free/built-in licenses is consistent with the installed edition, but the edition is now confirmed independently through the installation utility rather than inferred from the license list.

The WebPACK edition is device-limited, so the next part of T0.2 is to establish which target device families/devices are covered by this installation and compare that coverage against the programme's required device.

### Evidence

Screenshot: Xilinx/Vivado Select Edition screen showing:

Currently Installed Edition → Vivado HL WebPACK

## The screenshot also documents that upgrading to the Design or System Edition requires the appropriate licenses.


T0.2 — WebPACK Device Coverage

### Assumption

The installed Vivado HL WebPACK edition is device-limited. Before proceeding with the design environment, the target device must therefore be checked against the devices supported by WebPACK.

### Method

The target FPGA device was identified as:

xc7z020clg484-1

The device family and device number were then compared with the Vivado Design Suite 2019.1 Release Notes, UG973, under the WebPACK supported-device table.

### Result

The target device belongs to the Zynq-7000 SoC family and has the device identifier XC7Z020.

UG973 (v2019.1) lists the following Zynq-7000 devices as supported by Vivado WebPACK:

- XC7Z010

- XC7Z015

- XC7Z020

- XC7Z030

- XC7Z007S

- XC7Z012S

- XC7Z014S

The target XC7Z020 is therefore explicitly included in the WebPACK-supported device list.

### Conclusion

The installed Vivado HL WebPACK edition is compatible with the target device xc7z020clg484-1.

No Vivado Design Edition or System Edition upgrade is required solely for targeting this device.

### Evidence

- Vivado installation utility — Select Edition screen confirming:
Currently Installed Edition: Vivado HL WebPACK

- Xilinx UG973 v2019.1, Supported Devices section, confirming that XC7Z020 is supported by Vivado WebPACK.

## T0.3 — Numerical Computing Environment and Hardware Support

### Assumption

The numerical-computing environment required for the workflow must be identified and its installed hardware-generation components verified before attempting to establish the connection between the numerical environment and the FPGA design environment.

### Method

The installed MATLAB environment was inspected from the MATLAB Command Window.

The MATLAB version and installed products were queried using:

ver

The installed MATLAB hardware support packages were then checked using:

matlabshared.supportpkg.getInstalled

The results were recorded without installing or modifying any additional components.

### Result

The installed numerical-computing environment is:

- MATLAB R2026a Update 3

- MATLAB version 26.1.0.3276743

- Simulink 26.1 (R2026a)

- HDL Coder 26.1 (R2026a)

- HDL Verifier 26.1 (R2026a)

- MATLAB Coder 26.1 (R2026a)

The hardware support-package query returned:

No support packages installed.

Therefore, the base MATLAB installation contains HDL Coder and HDL Verifier, but the MATLAB hardware-support-package mechanism currently reports no installed hardware support packages.

### Conclusion

The numerical-computing environment is installed and the required HDL-generation products are present. However, no MATLAB hardware support packages are currently installed.

The connection/version binding between MATLAB R2026a and the installed Vivado 2019.1 environment has not yet been established. This remains a pending part of T0.3 and will be investigated separately rather than inferred from the installed-product list.

### Evidence

The following evidence is to be retained:

- t0_3_matlab_version.png — MATLAB version information

- t0_3_matlab_products.png — installed MATLAB/Simulink/HDL products

- t0_3_matlab_support_packages.png — result showing that no MATLAB hardware support packages are installed

### Open Investigation

The next step is to determine whether the installed MATLAB/HDL Coder environment can recognize or be configured to use the existing Vivado 2019.1 installation. The result of that investigation will determine whether the current environment satisfies the required version binding or whether an alternate approach is required.

## T0.3 — HDL Coder Vivado Tool-Path Configuration

### Assumption

The numerical-computing environment must be able to locate and configure the installed FPGA design tool before an HDL generation or FPGA implementation workflow can be attempted.

### Method

The installed Vivado 2019.1 executable was first invoked from MATLAB using:

!D:\xilinx\Vivado\2019.1\bin\vivado.bat -version

MATLAB successfully returned the Vivado 2019.1 version and build information.

The HDL Coder Vivado tool path was then configured for the current MATLAB session using:

hdlsetuptoolpath('ToolName','Xilinx Vivado', 'ToolPath','D:\xilinx\Vivado\2019.1\bin\vivado.bat')

No permanent installation or software modification was performed.

### Result

MATLAB successfully invoked the installed Vivado executable and returned:

- Vivado v2019.1 (64-bit)

- SW Build 2552052

- IP Build 2548770

HDL Coder then accepted the specified Vivado tool path and reported that the following paths were prepended to the system path:

D:\xilinx\Vivado\2019.1\bin

D:\xilinx\Vivado\2019.1\bin\..\lib\win64.o

No error was reported by the hdlsetuptoolpath command.

### Conclusion

The MATLAB R2026a environment can locate and execute the installed Vivado 2019.1 executable.

HDL Coder also accepts the Vivado 2019.1 tool path and configures the corresponding Vivado directories for the current MATLAB session.

This establishes that basic tool-path integration is functional.

However, this result does not by itself establish full workflow compatibility between MATLAB R2026a/HDL Coder and Vivado 2019.1. Full compatibility still needs to be verified by exercising the relevant HDL Coder FPGA workflow.

### Evidence

- t0_3_matlab_environment_and_support_packages.png — MATLAB R2026a Update 3 and installed MATLAB/HDL products, followed by the support-package check showing that no support packages are installed.

- t0_3_matlab_vivado_toolpath_configuration.png — MATLAB successfully invoking Vivado 2019.1 and HDL Coder successfully accepting the Vivado 2019.1 tool path and prepending the required Vivado directories to the system path.

### Status

Basic tool-path configuration: PASS

Full MATLAB R2026a ↔ Vivado 2019.1 workflow compatibility: PENDING VERIFICATION

# T0.4 — Embedded Linux Build System Availability

## Assumption

The P0 environment requires an embedded Linux build system. The purpose of this check is to determine whether the PetaLinux command-line environment is available on the current system.

## Method

The following command was executed from Command Prompt:

where petalinux-config

## Result

The command returned:

INFO: Could not find files for the given pattern(s).

No petalinux-config executable was found through the current Windows environment PATH.

## Conclusion

The PetaLinux build-system command was not found on the system during this check.

This establishes an availability gap only. The P0 guide requires functional proof by creating a project and completing a configuration step; that functional verification has not been performed.

## Evidence

Evidence file: T0_4_petalinux_command_not_found.png

The screenshot records the Command Prompt output showing that petalinux-config could not be found.

## Status

Availability check: NOT FOUND

Functional verification: NOT PERFORMED

Installation/remediation: PENDING

# T0.5 — Second-Vendor FPGA Design Suite Availability

## Assumption

The P0 verification matrix identifies a Second-vendor suite as a separate toolchain from the legacy and current primary FPGA design-suite versions. The required functional proof is synthesis of a trivial module followed by generation of a timing report.

The guide does not identify the second vendor by product name in the checked section, so no specific vendor was assumed.

## Method

Windows installed-software registry entries were searched for common FPGA second-vendor products using:

reg query "HKLM\SOFTWARE\Microsoft\Windows\CurrentVersion\Uninstall" /s /v DisplayName | findstr /i "Intel Altera Quartus Lattice Microchip Libero"

The 32-bit application registry location was also checked:

reg query "HKLM\SOFTWARE\WOW6432Node\Microsoft\Windows\CurrentVersion\Uninstall" /s /v DisplayName | findstr /i "Intel Altera Quartus Lattice Microchip Libero"

## Result

Both searches returned no matching installed-software entries.

No Intel/Altera/Quartus, Lattice, or Microchip/Libero installation entry was identified in the checked Windows uninstall registry locations.

## Conclusion

A second-vendor FPGA design suite was not found in the checked installed-software registry entries.

This does not by itself prove that no second-vendor software exists anywhere on the system; it establishes that no matching installation entry was found using the documented checks.

The required functional test has therefore not been performed.

## Evidence

Evidence file: T0_5_second_vendor_suite_not_found.png

The screenshot records the Command Prompt searches and their empty results.

## Status

Availability check: NOT FOUND

Functional verification: NOT PERFORMED

Installation/remediation: PENDING

# Pre-Installation Storage Assessment

## Assumption

Before installing the software components that were not found during the P0 environment inventory, the available disk space should be compared with a preliminary estimate of the storage required.

This assessment is supporting environment evidence and is not a numbered P0 task. The assignment does not define a specific storage requirement for the complete toolchain, so the values below are preliminary planning estimates.

## Method

The available disk space on the development machine was checked using Windows Command Prompt.

The recorded values were:

DeviceID  FreeSpace    Size

C:        30182326272  146104594432

D:        715722078?   109258469376

The verified approximate free space was:

C: ≈ 30.18 GB free

D: ≈ 71.57 GB free

Therefore, the combined available space is approximately:

30.18 GB + 71.57 GB = 101.75 GB

The currently identified missing tool categories were then considered for preliminary installation planning:

| Component | Rough planning estimate |
| --- | --- |
| Current Vivado + Vitis | ~100 GB |
| PetaLinux | ~30–50 GB |
| Second-vendor FPGA suite | ~10–25 GB |
| Preliminary total | ~140–175 GB |

These are planning estimates only. Exact installation requirements cannot yet be established because the exact current-tool release, PetaLinux release, and second-vendor product have not all been identified.

## Result

The system currently has approximately:

101.75 GB

of combined free space across C: and D:.

The preliminary estimate for the currently identified missing software is:

~140–175 GB

Therefore:

Available space:       ~101.75 GB

Preliminary required:  ~140–175 GB

The available space is below the preliminary estimated requirement.

This assessment also does not reserve additional space for temporary installation files, downloads, build products, or caches.

## Conclusion

The current machine has approximately 101.75 GB of combined free disk space.

The preliminary estimate indicates that approximately 140–175 GB may be required for the currently identified missing tool categories.

Therefore, additional storage capacity or storage cleanup will likely be required before installing the complete missing toolchain.

This is a preliminary planning result only. The final storage requirement should be recalculated after the exact software releases and products have been established.

## Evidence

Evidence file:

t0_storage_available_space.png

This evidence records the measured free space on the C: and D: drives.

The following existing evidence supports identification of the missing components:

- t0_1_current_vivado_vitis_not_found.png

- t0_4_petalinux_command_not_found.png

- t0_5_second_vendor_suite_not_found.png

## Status

Storage measurement: COMPLETED

Preliminary storage assessment: COMPLETED

Final installation-size calculation: PENDING exact software identification

Installation/remediation: NOT STARTED

## T0.6 — Repository and Generated-File Handling

### Assumption

The repository must contain the reproducible source/scripts while generated Vivado artefacts must remain outside version control.

### Method

The assignment workspace was inspected using Git and filesystem commands.

The Vivado directory was examined with:

find assignment/01_rtl/vivado -maxdepth 2 -type d

The command showed generated Vivado directories including:

p0_baseline.cache

p0_baseline.hw

p0_baseline.ip_user_files

The repository root was then inspected with ls -la.

The existing .gitignore was reviewed and updated so that generated Vivado/build artefacts were ignored while the project-generation script remained trackable.

git status was used to verify the resulting state.

### Result

The Vivado-generated directories were present in the working tree but were not intended to be version-controlled.

The project-generation script:

assignment/01_rtl/scripts/create_project.tcl

was detected as a source file and staged.

The .gitignore modification was staged separately as part of the same logical repository change.

After committing the change, the working tree was confirmed clean.

### Conclusion

The repository now distinguishes between reproducible source/scripts and generated Vivado artefacts.

The project-generation script is under version control, while generated project/build directories are excluded through .gitignore.

This establishes the initial configuration-control mechanism required for reproducible work.

### Evidence

- Vivado License Manager — View Host Information

- Vivado License Manager — Manage License Search Paths

- Vivado License Manager — View License Status